0                                                                         0

N

0                                                                         0

N

N

N

N

N

you will have to move your nose to approximately 20cm from the monitor but lined up both horizontally and vertically  with each picture you want to view your nose should be in line with the N between each photo.

look at the zeros at the top of each picture and cross your eyes until both zero’s are lined up, you then will find that you have 3 pictures in front of you, but the centre picture is 3D, the top picture is greater depth and the distance to the power pole on the right of screen is noticeably halfway between the houses at the rear of shot and the camera,

the lower photo shows depth using the fir trees versus the houses in the background.

To take your own 3d photos with a single camera you need to use a fine tipped marker pen and if your camera has an lcd display viewfinder, place a tiny dot in the centre of the viewfinder screen, this is most important as this is your focus point.

To view back the 3d photo, you need to do as I did here, but you need to reverse the order in which you took them. So the second photo is on the left and the first photo is on the right.

Non moving targets are the only 3D shots you can take (unless you can get your subject to freeze totally between shots), simply aim the camera at your target, placing the dot you made on an object in the distance that WON’T move e.g. A mountain top or the tip of a power pole like I did in the top photo. Take your shot. Move the camera 6cm to the right, put the dot back where you took the first photo, and take the second shot. You must never change the height when you do this, as the 3d photo will fail. You are simply imitating the way we see. Two identical images, 6cm apart is what we see normally, by crossing our eyes to view the 2 photos, we trick our brain into seeing what both our eyes would see in real life, the left eye views what the left eye would see, and the right eye views the what the right eye would see, but by crossing our eyes, we get the depth, an equation the brain works out from our eye triangulation.